# After the Fire: Soil Management

After a wildfire, soils can be impacted in a number of ways, including chemical and heavy metal contamination, alteration of nutrients, organic matter content and pH, an increase in erosion hazard, and altered water holding capacity. The cumulative effects of these changes can modify soil productivity and biological diversity, as well as pose health risks to humans and animals. Urban soils in particular are at an increased risk for chemical and heavy metal contamination post-fire because households tend to have features and contain products that leach these contaminants when impacted by fire. Examples of potential contaminants include debris from lead-based paint and pipes, common household chemicals, pesticides, and petroleum-based products. All of these contaminants have the potential to impact soil and human health and the organisms that interact with it.

## Understand Your Site History

When evaluating your soil health post-wildfire, the first thing to do is understand the history of your site. One way to learn about what may have impacted a site is to visually inspect the area, as it may provide some evidence. For example, if the site is next to buildings built before 1978, lead may be a potential hazard, especially if you see that the paint is chipped or peeling. Past land use of the site can be  accessed throughthroughaccessed throughaccessed through public records online or at the library, consulting Sanborn maps and aerial photographs of the area from the past, or asking neighbors who may have been there before you. If soroundingsurroundingsorounding harzdous materials are identified as potentital sources of contamination, submitingsubmittingsubmiting a soil sample for chemical analysis will help determine the severity of the contamination.

## Testing Your Soil

Testing is the next thing that needs to happen in order to understand how wildfire may have affected your soil. Laboratory soil testing is recommended, especially if the soil is currently being used or going to be used for urban agriculture. Residents impacted by fire can access post-fire soil test results from their clean-up contractor or seek out testing themselves. While DIY soil test kits sold at retail stores can sometimes provide basic information,, lab tests yield reliable and precise results. To learn more about lab-based and at-home soil testing options, visit UC ANR’s Healthy Soils for a Healthy California website (https://ucanr.edu/sites/soils/ ) or the California Environmental Laboratory Accreditation Program (ELAP; https://www.waterboards.ca.gov/drinking_water/certlic/labs/).

Figure 1: Collecting a composite soil sample. PC: Surls et al. 2016

When collecting soil samples to send out for testing, youyouyou may consider takingtake separate samples from locations with different characteristics throughout the property , label each sample bag,and map them for your records (Figure 1). This can help determine if any potential contaminants found are localized or widespread across the site. Each sample should be composed of 15-20 subsamples collected throughout the sampling area, mixed thoroughly oninon a bucket and placing about one pound of soil in a clean bag labeled appropriately. (Figure 1).. Should each sample be placed in its own bag?  Do you label the bags?  Is all of this information on the test kit?  What PPE should someone wear while testing their soil (put a reference to BMP 1. Below)?

The U.S. Environmental Protection Agency (U.S. EPA) recommends that for urban areas, “at a minimum, the soil test should include pH, percent organic matter, nutrients, micronutrients, and metals, including lead” (U.S. EPA 2011b). This level of testing is adequate for a site that has been residential or a green space. Most commercial soil labs can test for the most important heavy metals, including lead, arsenic, cadmium, chromium, and nickel.

More testing may be needed for sites with a history of industrial or commercial use. These sites have the potential to be contaminated by a wider range of heavy metal and polycyclic aromatic hydrocarbon (PAH) contaminants that will require more extensive testing to detect. An EPA-recognized laboratory is the best choice for this level of testing.

## Ways to Remediate Your Soil

After determining the history and state of your soil, the next steps will depend on what soil testing results indicate. If your soil testing indicates that no contaminants have been found exceeding allowable levels, no further action is needed. If your soil testing indicates that contaminants have been found at or exceeding allowable levels, consider the following remediation strategies:

### Work with Your Existing Soil

Working with existing soil is most feasible for sites with no history of industrial use and whose soil test results came back with moderate levels of contaminants. Existing soil should be diluted, maintained at a neutral pH, and amended with appropriate additions of organic matter.

- Contaminants are often concentrated in the top two inches of soil and can be diluted by digging and mixing the soil deeply and thoroughly.

- Soil pH should be maintained at or close to neutral (6.5-7.0). Soil nutrients are most available to plants at this pH, while lead and some other heavy metals are less available.

- Add organic matter such as high-quality compost to the soil and continue to do so with each planting that follows. Heavy metal contaminants will bind to organic matter and become less available to plants. Organic matter also improves soil structure, infiltration, and water-holding capacity, creating a better environment for plant roots. Additionally, biochar and charcoal can be used to improve existing soil, however, adding these can raise soil pH, which could pose a problem if the soil being treated is already alkaline from exposure to ash and other contaminants.

### Use Raised Beds or Other Containers

A common approach for sites with contaminated soil is to install or build raised beds and fill them with clean soil. If building your own frames, use sturdy materials that won’t leach contaminants into the soil, like redwood or other non-treated lumber, brick, concrete, or rocks. Water-permeable fabrics can be applied as a barrier between the existing site soil and the imported soil going into the beds.

- When importing soil, consider purchasing topsoil or planting mix certified by the Organic Materials Review Institute (OMRI). Local urban farmers or gardeners may have additional recommendations. Some vendors may be able to provide documentation that soil has been tested. If not, you can send soil samples to the lab for testing.

- Other types of containers besides raised beds can be used as well. Some commercially available examples include growing systems that would be appropriate for smaller-scale projects, and mesh “socks” that are filled with growing medium and placed on top of the soil. Avoid using items as planting containers that could contaminate soil as they degrade over time (e.g., tires).

### Remove Contaminated Soil

Soil removal is the most common practice and the most extreme when dealing with contaminated soil. This process involves removing the existing soil from a site and replacing it with soil that is certified safe. The contaminated soil should be disposed of according to law. While this strategy is helpful for assuaging fears of contamination, it isn’t necessarily accessible, as it is an expensive process to excavate soil, remove it, and replace it.

## Best Management Practices for Near-Home Garden Soil

If you are concerned about inhaling or ingesting chemical contaminants from plants, produce, or soil, use the following best management practices for interacting with soil that has been potentially affected by wildfire:

- Wear gloves, boots, eye protection, a long-sleeved shirt and long pants when interacting with contaminated soil to prevent ingestion and direct contact.

- Use raised garden beds and import clean soils to avoid inhalation, ingestion, and plant intake of contaminants over time.

- Amend soil with clean, high-quality compost (OMRI) to bind and dilute potential contaminants. High-phosphate compost will bind and dilute potential contaminants, particularly heavy metals.

- Use mulch to cover the soil to prevent airborne soil and dust up-splash.

- Promote good drainage and use drip irrigation to prevent up-splash, particularly at the bottom of slopes that burned and after long wet periods. Contaminants can accumulate in these areas under wet conditions.

- Be mindful not to track contaminated soil into your home.

- Wash hands before and after harvest and wash your produce before it is consumed. White vinegar can be used to lift soil particles off produce, particularly for leafy greens and root crops.

References

Bennaton, R. (2020). Safe soils after fires. YouTube. https://www.youtube.com/watch?v=q25_FEwM2rY.

Casale, R. (2020). Understanding wildfire impacts. YouTube. https://www.youtube.com/watch?v=u9V9QIrra4Q.

Morelli, J.G. (2020). Sonoma County Complex Fires October 2017 site clearance and testing requirements. YouTube. https://www.youtube.com/watch?v=oS2E-tlSL38.

O’Brien, C. (2020). Contaminants in soil after fire. YouTube. https://www.youtube.com/watch?v=Yhtg_mPEpSs.

Surls, R., Borel, V., Biscaro, A. (2016). Soils in urban agriculture: testing, remediation, and best management practices. UC ANR Publication 8552. https://anrcatalog.ucanr.edu/pdf/8552.pdf.

UC Cooperative Extension. (n.d.). Groups of contaminants that can be found in urban soils. https://ucanr.edu/sites/SoCo/files/338300.pdf.

UC Cooperative Extension. (2025, February 1). Food Safety After Urban Wildfire. https://cesonoma.ucanr.edu/Disaster_Resources/Fire/Produce_Safety_after_Urban_Wildfire/.

Authored by Isabella Zahra, UC ANR Fire Network Staff Research Associate

Katie Low, UC ANR Fire Network Coordinator

With additional feedback from

INSERT REVIEWERS NAMES HERE

Learn more about the UC ANR Fire Network by visiting our webpage at https://ucanr.edu/sites/fire/