## WEEKLY CROP WATER USE
Based on local CIMIs Weather Stations (in inches)

### (Estimated Crop Evapotranspiration or ETc)

### 6/19/2026 through 6/25/2026

#### #148 Merced

| Crops (Leaf out date) | 06/19 - 06/25 Water Use | Accumulated Seasonal Water Use | 06/26 - 07/02 Estimated ETc |
| --- | --- | --- | --- |
| Almonds (3/6) * | 1.94 | 18.17 | 2.03 |
| Citrus (2/1) | 1.41 | 17.54 | 1.47 |
| Pistachio (4/1) * ** | 2.11 | 13.77 | 2.17 |
| Raisin Grapes (3/9) (11 ft. row spacing) | 1.62 | 12.29 | 1.68 |
| Stone Fruit (3/16) | 1.76 | 12.95 | 1.85 |
| Walnuts (4/1) | 1.90 | 13.79 | 2.04 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | 1.80 | 12.94 | 1.89 |
| Past 7 days precipitation (inches) |  | 0.00 |  |
| Accumulated precipitation (inches) (01/01/2026) |  | 0.00 |  |

#### #39 Parlier

| Crops (Leaf out date) | 06/19 - 06/25 Water Use | Accumulated Seasonal Water Use | 06/26 - 07/02 Estimated ETc |
| --- | --- | --- | --- |
| Almonds (3/6) * | 2.00 | 19.36 | 1.96 |
| Citrus (2/1) | 1.48 | 18.88 | 1.40 |
| Pistachio (4/1) * ** | 2.17 | 14.29 | 2.10 |
| Raisin Grapes (3/9) (11 ft. row spacing) | 1.64 | 12.76 | 1.61 |
| Stone Fruit (3/16) | 1.82 | 13.58 | 1.78 |
| Walnuts (4/1) | 1.96 | 14.29 | 1.97 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | 1.83 | 13.61 | 1.82 |
| Past 7 days precipitation (inches) |  | 0.00 |  |
| Accumulated precipitation (inches) (01/01/2026) |  | 4.94 |  |

#### #258 Lemon Cove

| Crops (Leaf out date) | 06/19 - 06/25 Water Use | Accumulated Seasonal Water Use | 06/26 - 07/02 Estimated ETc |
| --- | --- | --- | --- |
| Almonds (3/6) * | 1.96 | 19.00 | 1.97 |
| Citrus (2/1) | 1.45 | 18.74 | 1.41 |
| Pistachio (4/1) * ** | 2.13 | 13.92 | 2.11 |
| Raisin Grapes (3/9) (11 ft. row spacing) | 1.63 | 12.48 | 1.62 |
| Stone Fruit (3/16) | 1.78 | 13.34 | 1.79 |
| Walnuts (4/1) | 1.92 | 14.00 | 1.98 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | 1.80 | 13.31 | 1.83 |
| Past 7 days precipitation (inches) |  | 0.00 |  |
| Accumulated precipitation (inches) (01/01/2026) |  | 4.96 |  |

Dates in parentheses above indicate leaf out or starting date for ET accumulation for the specific crop.

* Estimates are for orchard floor conditions where vegetation is managed by some combination of strip applications of herbicides, frequent mowing or tillage, and by mid and late season shading and water stress.  Weekly estimates of soil moisture loss can be as much as 25 percent higher in orchards where cover crops are planted and managed more intensively for maximum growth.

** Very vigorous, non-salt affected peak season pistachio Kc can be as high as 1.19 – resulting in about 8% greater water use than shown in these tables.

### PAST WEEKLY APPLIED WATER IN INCHES, ADJUSTED FOR EFFICIENCY1

#### #148 Merced

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds (3/6) * | 3.0 | 2.6 | 2.3 | 2.0 |
| Citrus (2/1) | 2.2 | 1.9 | 1.7 | 1.5 |
| Pistachio (4/1) * ** | 3.2 | 2.8 | 2.5 | 2.2 |
| Raisin Grapes (3/9) (11 ft. row spacing) | NA | NA | NA | 1.7 |
| Stone Fruit (3/16) | 2.7 | 2.3 | 2.1 | 1.9 |
| Walnuts (4/1) | 2.9 | 2.5 | 2.2 | 2.0 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | NA | NA | NA | 1.9 |

#### #39 Parlier

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds (3/6) * | 3.1 | 2.7 | 2.4 | 2.1 |
| Citrus (2/1) | 2.3 | 2.0 | 1.7 | 1.6 |
| Pistachio (4/1) * ** | 3.3 | 2.9 | 2.6 | 2.3 |
| Raisin Grapes (3/9) (11 ft. row spacing) | NA | NA | NA | 1.7 |
| Stone Fruit (3/16) | 2.8 | 2.4 | 2.1 | 1.9 |
| Walnuts (4/1) | 3.0 | 2.6 | 2.3 | 2.1 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | NA | NA | NA | 1.9 |

#### #258 Lemon Cove

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds (3/6) * | 3.0 | 2.6 | 2.3 | 2.1 |
| Citrus (2/1) | 2.2 | 1.9 | 1.7 | 1.5 |
| Pistachio (4/1) * ** | 3.3 | 2.8 | 2.5 | 2.2 |
| Raisin Grapes (3/9) (11 ft. row spacing) | NA | NA | NA | 1.7 |
| Stone Fruit (3/16) | 2.7 | 2.4 | 2.1 | 1.9 |
| Walnuts (4/1) | 3.0 | 2.6 | 2.3 | 2.0 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | NA | NA | NA | 1.9 |

1 The amount of water required by a specific irrigation system to satisfy evapotranspiration. Typical ranges in irrigation system efficiency are: Drip, 80%-95%; Micro-sprinkler, 80%-90%; Sprinkler, 70%-85%; and Border-furrow, 50%-75%.

### PAST WEEKLY APPLIED WATER IN GALLONS PER TREE OR VINE, ADJUSTED FOR EFFICIENCY

#### #148 Merced

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds 115 Trees/A | 708.4 | 613.9 | 543.1 | 472.2 |
| Citrus 110 Trees/A | 543.1 | 469.0 | 419.7 | 370.3 |
| Pistachio 106 Trees/A | 819.7 | 717.3 | 640.4 | 563.6 |
| Raisin Grapes 566 Vines/A | NA | NA | NA | 81.6 |
| Stone Fruit 172 Trees/A | 426.3 | 363.1 | 331.5 | 300.0 |
| Walnuts 76 Trees/A | 1,036.1 | 893.2 | 786.0 | 714.6 |
| Winegrapes 622 Vines/A | NA | NA | NA | 82.9 |

#### #39 Parlier

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds 115 Trees/A | 732.0 | 637.5 | 566.7 | 495.9 |
| Citrus 110 Trees/A | 567.8 | 493.7 | 419.7 | 395.0 |
| Pistachio 106 Trees/A | 845.4 | 742.9 | 666.0 | 589.2 |
| Raisin Grapes 566 Vines/A | NA | NA | NA | 81.6 |
| Stone Fruit 172 Trees/A | 442.0 | 378.9 | 331.5 | 300.0 |
| Walnuts 76 Trees/A | 1,071.9 | 929.0 | 821.8 | 750.3 |
| Winegrapes 622 Vines/A | NA | NA | NA | 82.9 |

#### #258 Lemon Cove

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds 115 Trees/A | 708.4 | 613.9 | 543.1 | 495.9 |
| Citrus 110 Trees/A | 543.1 | 469.0 | 419.7 | 370.3 |
| Pistachio 106 Trees/A | 845.4 | 717.3 | 640.4 | 563.6 |
| Raisin Grapes 566 Vines/A | NA | NA | NA | 81.6 |
| Stone Fruit 172 Trees/A | 426.3 | 378.9 | 331.5 | 300.0 |
| Walnuts 76 Trees/A | 1,071.9 | 929.0 | 821.8 | 714.6 |
| Winegrapes 622 Vines/A | NA | NA | NA | 82.9 |

For further information concerning all counties receiving this report, contact the Fresno Co. Farm Advisor's office at (559) 241-7526.

## WEEKLY CROP WATER USE
Based on local CIMIs Weather Stations (in inches)

### (Estimated Crop Evapotranspiration or ETc)

### 6/19/2026 through 6/25/2026

#### #124 Panoche

| Crops (Leaf out date) | 06/19 - 06/25 Water Use | Accumulated Seasonal Water Use | 06/26 - 07/02 Estimated ETc |
| --- | --- | --- | --- |
| Almonds (3/6) * | 2.07 | 19.96 | 2.10 |
| Citrus (2/1) | 1.54 | 19.13 | 1.54 |
| Pistachio (4/1) * ** | 1.90 | 12.82 | 2.24 |
| Raisin Grapes (3/9) (11 ft. row spacing) | 1.70 | 13.45 | 1.68 |
| Stone Fruit (3/16) | 1.89 | 14.23 | 1.92 |
| Walnuts (4/1) | 2.03 | 15.12 | 2.11 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | 1.61 | 12.13 | 1.96 |
| Past 7 days precipitation (inches) |  | 0.00 |  |
| Accumulated precipitation (inches) (01/01/2026) |  | 4.49 |  |

#### #2 Five Points

| Crops (Leaf out date) | 06/19 - 06/25 Water Use | Accumulated Seasonal Water Use | 06/26 - 07/02 Estimated ETc |
| --- | --- | --- | --- |
| Almonds (3/6) * | 2.15 | 20.65 | 2.21 |
| Citrus (2/1) | 1.59 | 19.96 | 1.61 |
| Pistachio (4/1) * ** | 1.97 | 12.72 | 2.31 |
| Raisin Grapes (3/9) (11 ft. row spacing) | 1.77 | 13.47 | 1.75 |
| Stone Fruit (3/16) | 1.97 | 14.44 | 2.00 |
| Walnuts (4/1) | 2.11 | 15.14 | 2.18 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | 1.67 | 12.19 | 2.03 |
| Past 7 days precipitation (inches) |  | 0.00 |  |
| Accumulated precipitation (inches) (01/01/2026) |  | 2.33 |  |

#### #15 Stratford

| Crops (Leaf out date) | 06/19 - 06/25 Water Use | Accumulated Seasonal Water Use | 06/26 - 07/02 Estimated ETc |
| --- | --- | --- | --- |
| Almonds (3/6) * | 2.11 | 21.40 | 2.21 |
| Citrus (2/1) | 1.55 | 20.90 | 1.61 |
| Pistachio (4/1) * ** | 2.28 | 15.47 | 2.31 |
| Raisin Grapes (3/9) (11 ft. row spacing) | 1.74 | 13.98 | 1.75 |
| Stone Fruit (3/16) | 1.93 | 14.92 | 2.00 |
| Walnuts (4/1) | 2.07 | 15.72 | 2.18 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | 1.93 | 14.88 | 2.03 |
| Past 7 days precipitation (inches) |  | 0.00 |  |
| Accumulated precipitation (inches) (01/01/2026) |  | 2.25 |  |

Dates in parentheses above indicate leaf out or starting date for ET accumulation for the specific crop.

* Estimates are for orchard floor conditions where vegetation is managed by some combination of strip applications of herbicides, frequent mowing or tillage, and by mid and late season shading and water stress.  Weekly estimates of soil moisture loss can be as much as 25 percent higher in orchards where cover crops are planted and managed more intensively for maximum growth.

** Very vigorous, non-salt affected peak season pistachio Kc can be as high as 1.19 – resulting in about 8% greater water use than shown in these tables.

### PAST WEEKLY APPLIED WATER IN INCHES, ADJUSTED FOR EFFICIENCY1

#### #124 Panoche

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds (3/6) * | 3.2 | 2.8 | 2.4 | 2.2 |
| Citrus (2/1) | 2.4 | 2.1 | 1.8 | 1.6 |
| Pistachio (4/1) * ** | 2.9 | 2.5 | 2.2 | 2.0 |
| Raisin Grapes (3/9) (11 ft. row spacing) | NA | NA | NA | 1.8 |
| Stone Fruit (3/16) | 2.9 | 2.5 | 2.2 | 2.0 |
| Walnuts (4/1) | 3.1 | 2.7 | 2.4 | 2.1 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | NA | NA | NA | 1.7 |

#### #2 Five Points

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds (3/6) * | 3.3 | 2.9 | 2.5 | 2.3 |
| Citrus (2/1) | 2.4 | 2.1 | 1.9 | 1.7 |
| Pistachio (4/1) * ** | 3.0 | 2.6 | 2.3 | 2.1 |
| Raisin Grapes (3/9) (11 ft. row spacing) | NA | NA | NA | 1.9 |
| Stone Fruit (3/16) | 3.0 | 2.6 | 2.3 | 2.1 |
| Walnuts (4/1) | 3.2 | 2.8 | 2.5 | 2.2 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | NA | NA | NA | 1.8 |

#### #15 Stratford

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds (3/6) * | 3.2 | 2.8 | 2.5 | 2.2 |
| Citrus (2/1) | 2.4 | 2.1 | 1.8 | 1.6 |
| Pistachio (4/1) * ** | 3.5 | 3.0 | 2.7 | 2.4 |
| Raisin Grapes (3/9) (11 ft. row spacing) | NA | NA | NA | 1.8 |
| Stone Fruit (3/16) | 3.0 | 2.6 | 2.3 | 2.0 |
| Walnuts (4/1) | 3.2 | 2.8 | 2.4 | 2.2 |
| Winegrapes (3/9) (10 ft. spacing on California Sprawl Trellis) *** | NA | NA | NA | 2.0 |

1 The amount of water required by a specific irrigation system to satisfy evapotranspiration. Typical ranges in irrigation system efficiency are: Drip, 80%-95%; Micro-sprinkler, 80%-90%; Sprinkler, 70%-85%; and Border-furrow, 50%-75%.

### PAST WEEKLY APPLIED WATER IN GALLONS PER TREE OR VINE, ADJUSTED FOR EFFICIENCY

#### #124 Panoche

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds 115 Trees/A | 755.6 | 661.1 | 566.7 | 519.5 |
| Citrus 110 Trees/A | 592.5 | 518.4 | 444.3 | 395.0 |
| Pistachio 106 Trees/A | 742.9 | 640.4 | 563.6 | 512.3 |
| Raisin Grapes 566 Vines/A | NA | NA | NA | 86.4 |
| Stone Fruit 172 Trees/A | 457.8 | 394.7 | 347.3 | 315.7 |
| Walnuts 76 Trees/A | 1,107.6 | 964.7 | 857.5 | 750.3 |
| Winegrapes 622 Vines/A | NA | NA | NA | 74.2 |

#### #2 Five Points

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds 115 Trees/A | 779.2 | 684.8 | 590.3 | 543.1 |
| Citrus 110 Trees/A | 592.5 | 518.4 | 469.0 | 419.7 |
| Pistachio 106 Trees/A | 768.5 | 666.0 | 589.2 | 538.0 |
| Raisin Grapes 566 Vines/A | NA | NA | NA | 91.2 |
| Stone Fruit 172 Trees/A | 473.6 | 410.5 | 363.1 | 331.5 |
| Walnuts 76 Trees/A | 1,143.3 | 1,000.4 | 893.2 | 786.0 |
| Winegrapes 622 Vines/A | NA | NA | NA | 78.6 |

#### #15 Stratford

| System Efficiency>> | 65% | 75% | 85% | 95% |
| --- | --- | --- | --- | --- |
| Almonds 115 Trees/A | 755.6 | 661.1 | 590.3 | 519.5 |
| Citrus 110 Trees/A | 592.5 | 518.4 | 444.3 | 395.0 |
| Pistachio 106 Trees/A | 896.6 | 768.5 | 691.7 | 614.8 |
| Raisin Grapes 566 Vines/A | NA | NA | NA | 86.4 |
| Stone Fruit 172 Trees/A | 473.6 | 410.5 | 363.1 | 315.7 |
| Walnuts 76 Trees/A | 1,143.3 | 1,000.4 | 857.5 | 786.0 |
| Winegrapes 622 Vines/A | NA | NA | NA | 87.3 |

For further information concerning all counties receiving this report, contact the Fresno Co. Farm Advisor's office at (559) 241-7526.